ENERGYMIND TUNISIA

# WattWitness

Microgrid Data Trust Layer

Formal Project Proposal · Phase 1: Ideation & Data Exploration

Chosen system slice: one simulated microgrid node
PV + wind + variable synthetic building load + battery + grid connection

Version 1.0  |  15 October 2026

Proposal principle

Build and demonstrate a transparent, human-supervised trust layer between simulated energy telemetry and any later automated decision. Report what is implemented, what remains a design, and what still requires validation.

# 1. Executive summary

WattWitness is a Phase 1 prototype for the EnergyMind Tunisia challenge. It treats every telemetry reading as testimony and cross-examines it against the physical relationships it must satisfy and against simple data-quality rules. Each alert carries a plain-language reason and an evidence score and goes to a human reviewer, whose decision is recorded in an audit log. The goal is not to replace an operator: the system should state what its evidence supports and escalate uncertain cases.

The prototype uses a one-year hourly simulation-derived dataset. PV and wind values are retained from a supplied base simulation export; the building load and context labels are synthetic; grid/battery dispatch is recalculated under stated assumptions; and voltage/current/inverter channels are derived through simplified formulas. A meter model then adds realistic measurement error, finite resolution and telemetry outages, so the data are not artificially perfect. Controlled faults are injected into a held-out window to test detection. Those results are simulated test results, not evidence of real-world performance.

# 2. Scope

| In scope for Phase 1 | Out of scope / future work |
| --- | --- |
| One simulated microgrid telemetry node; hourly series for PV, wind, building load, grid import, battery and derived electrical signals. | Production deployment, real-time connection to utility infrastructure, autonomous equipment control, or automatic data repair. |
| Controlled injection of sensor/data-quality faults with traceable event metadata. | Claims that injected events replicate every real equipment failure or cyberattack. |
| Transparent rules and physical-consistency checks; comparisons against baselines. | A fully validated ML model or calibrated probability of fault. |
| Human review queue with recorded decisions and an escalation policy. | Multi-node exchange or federated learning implementation; Phase 1 includes design documentation only. |

# 3. Objectives

Detect selected classes of missing, abrupt, frozen, delayed, drifting and biased telemetry in a repeatable experiment.

Compare suspicious readings with related power, voltage and current signals to surface inconsistencies.

Give each alert a plain-language rationale, evidence strength, recommended check and human-review state.

Define a human review step in which decisions and reasons are recorded and no equipment is controlled.

Define clear escalation boundaries and propose a privacy-preserving mechanism for future sharing between nodes.

Deliver the prepared dataset, source code, experiment results, technical report and documented limitations.

# 4. Data and anomaly-simulation plan

## 4.1 Baseline dataset

The prepared dataset contains 8,784 hourly rows from 1 June 2023 through 31 May 2024. The renewable PV/wind series are retained from the supplied simulation export. The original repeated daily load is used only as a rough shape template; a different synthetic building load is generated with weekday/weekend differences, seasonal multipliers, daily variability, correlated hourly noise and intermittent demand events. EV load is excluded.

Because derived channels satisfy their equations exactly, a seeded meter model converts this ideal state into measured telemetry: about 0.5% error on power and current, 0.2% on voltage, finite reporting resolution, and 18 natural outages leaving 39 rows with unknown values. Detection limits are learned from the training period of this measured data.

| Signal family | Construction and interpretation |
| --- | --- |
| PV and wind power | Retained from the supplied base simulation series; units and measurement/simulation provenance are inherited and still need confirmation. |
| Building load and context | Synthetic variable building demand plus generated context labels. Not measured demand or activity. |
| Grid and battery | Recalculated using a stated balance equation, 100 kW battery limit, assumed 100–500 kWh stored-energy range and 95% charge/discharge efficiency. |
| Voltage, current and inverter | Derived with simplified formulae using assumed 400 V three-phase voltage and power factors. These are not real sensor measurements or detailed equipment simulations. |

## 4.2 Injection protocol

The final 60 days of the prepared dataset are held out. A fixed seed creates 18 traceable events (three per type, from gross to subtle severity) altering 159 hourly readings; only readings whose value actually changed are labelled. Each injected event records its type, target signal, start/end times, duration, severity and seed. The untouched baseline is retained separately; injection labels are excluded from the detector inputs and are used only to evaluate predictions.

| Injected scenario | Demonstration intent |
| --- | --- |
| Spike in grid power | Detect a one-sample extreme jump. |
| Frozen PV reading | Detect a meaningful signal remaining unchanged for too long. |
| Gradual building-load drift | Test a slow offset against the modelled cross-signal relationships. |
| Missing battery SOC | Detect missing telemetry values. |
| Delayed/replayed PV signal | Test a reading that is plausible in magnitude but misaligned in time. |
| Plausible grid-current bias | Test an altered measurement that can conflict with voltage/power consistency. |

## 4.3 Evaluation

Compare simple rules, physical-consistency checks, their transparent combination, and an Isolation Forest baseline on the same held-out test window. Report precision, recall, F1 and average precision, event-level detection, recall by fault type, alerts on the untouched window, and sensitivity to meter error. The untouched-window figure is described as an alert rate, not a verified false-positive rate, because there are no independent labels for every real event in the base simulation.

# 5. Preliminary design for each official Challenge Goal

The four goal names below follow the official EnergyMind Tunisia challenge description. The implementation status is stated conservatively; a preliminary design is not the same as a completed implementation.

## Goal 1 — Anomaly & Bias Detection

Design: monitor the available energy variables and derive checks for missing values, abrupt jumps, frozen values and disagreement between linked electrical channels. The hybrid alert is raised when a data-quality rule triggers or a physical-consistency residual exceeds the current threshold. The controlled experiment covers spike, frozen value, gradual drift, missing value, delayed/replayed signal and plausible current bias.

Distinguishing event types: a reading that conflicts with related channels supports a data-quality hypothesis. A coherent change across multiple channels without a strong physical mismatch is shown as a potential operating event requiring context confirmation. In Phase 1 the system cannot establish causality with certainty because the simulation lacks independent labels for genuine grid events and the synthetic context is not a trusted operational log. Power-quality waveforms and sub-second smart-meter streams are outside the included hourly dataset, so the present implementation focuses on the signals actually available.

Status: partially implemented. The single-node detector and controlled test are implemented. Context-aware discrimination between genuine events, natural equipment failures and telemetry corruption requires stronger context data and independently labelled cases.

## Goal 2 — Confidence Scores & Explainability

Design: each alert carries a plain-language reason, the related residuals/rules, a qualitative evidence label, a suggested operator check, and an audit-row timestamp. Each alert also carries a simple heuristic evidence score and the size of every mismatch relative to its learned limit. The score is explicitly labelled as uncalibrated and is not presented as the probability that a fault occurred.

Status: partially implemented. Explanations and logging are generated. Confidence calibration, SHAP/LIME analysis, and independent human scoring of explanation quality are not yet completed.

## Goal 3 — Human-AI Escalation & Governance

Design: the AI may flag and explain, but will not change raw readings or control equipment. A human reviewer can confirm a data issue, judge that a change may be a genuine operating event, dismiss an alert, or escalate it, while recording the reason. Decisions are appended to an audit log whose existing entries are never rewritten. To reduce over-trust, the score is labelled as heuristic and shows the evidence and limitations; to reduce under-trust, it makes the checks and supporting signals inspectable and gives the operator a documented override/escalation path.

| Provisional evidence tier | Proposed response in Phase 1 |
| --- | --- |
| No meaningful rule or physical conflict; heuristic score below 0.35 | No automatic action. Continue monitoring; do not treat the lack of an alert as proof of safety. |
| Rule pattern without a strong physical mismatch; score around 0.35–0.69 | Queue for operator review. Ask the operator to inspect adjacent values, timing and context. |
| Strong physical mismatch; score 0.70 or higher | Prioritize human review and compare related signals/source records. Do not auto-correct or trip equipment. |
| Missing critical signal, contradictory evidence or uncertain diagnosis | Escalate to an authorized operator; preserve the raw data and log the decision. |

These score bands are illustrative governance thresholds for the demo, not calibrated values or a safety-certified policy. Automatic corrective actions remain disabled. Status: review workflow prototyped; production governance remains future work.

## Goal 4 — Shared Learning Across the Noosphere

Design: nodes may share approved anonymized anomaly patterns, aggregated calibration insights and relevant knowledge as normalized summaries (signal family, coarse shape/duration, checks triggered, detector version and reviewed outcome), not raw traces, precise timestamps, device identifiers or private schedules by default. Each receiving node checks units, signals, sign conventions and physical compatibility; patterns run in shadow mode and require provenance and review before adoption. The design considers privacy leakage, poisoned knowledge, negative transfer and false confidence.

Status: design documented only. No multi-node exchange, shared model or federated training is implemented in Phase 1; the challenge states that multi-node sharing may remain a documented design.

# 6. System architecture and operator workflow

Prepare the one-year simulated dataset; leave the clean source table unchanged.

Apply the meter model to obtain measured telemetry with realistic error and gaps.

Create a labeled faulted copy of the held-out test window using fixed injection settings.

Run data-quality rules and simplified physical-consistency checks; compare against baselines.

Generate alert explanations, evidence labels, metrics and a review queue.

A reviewer inspects each alert with its related signals and records a decision and a reason.

Retain a local audit record of reviewer label, decision and notes. No equipment command is sent.

# 7. Implementation status and deliverables

| Phase 1 element | Status |
| --- | --- |
| Prepared simulated dataset and data dictionary | Included; base export is not included in the public bundle. Confirm rights for derived PV/wind series before public upload. |
| Fault injection, detector and evaluation scripts | Implemented; repeatable with included prepared dataset. |
| Metric output and plots | Included; results are on controlled synthetic faults only. |
| Explainable alert output and review queue | Implemented; evidence score is heuristic, not calibrated. |
| Human review queue and decision log | Implemented as a prototype; not a secured production audit system. |
| Cross-node privacy/sharing approach | Documented design only, as permitted for Phase 1. |
| Formal proposal and technical report | Included as Word and PDF. |

# 8. Main risks and next validation steps

Verify inherited PV/wind provenance, units and redistribution rights for the prepared dataset.

Verify the battery-state interpretation, dispatch assumptions and sign convention against the originating simulation model.

Add examples of coherent genuine operating events and independently labelled sensor faults; evaluate event-versus-data classification.

Calibrate evidence scores on labelled data and assess explanation quality with operators.

If authorized, test replayed or non-invasive data with an independent laboratory node; do not connect the prototype to live control equipment.

For multi-node learning, implement sender identity, versioned schema, provenance checks, privacy tests and shadow-mode evaluation before activation.